Note on Second Phase "All-out Anti-mosquito Operations"

The Second Phase “All-out Anti-mosquito Operations” (the Operations) will be commenced on 2 May 2024 and all departments are requested to start the Operations simultaneously to suppress the density of adult mosquitoes and eliminate the potential mosquito breeding places prior to the rainy season.  All concerned departments will conduct the anti-mosquito work within their venue until the end of the rainy season (around October/November 2024) so as to sustain the effect of the Operations.

The Operations will involve the following important enhanced mosquito control measures -

- Carry out weekly inspection of the venues for detection and elimination of mosquito breeding places by physical removal of the water/ water bodies or application of larvicides; and

- Trimming of grass (if necessary) to discourage resting of the adult mosquitoes on the site and for easy detection of mosquito breeding places.

- Install Mosquito Trapping Device (MTD) (e.g. In2Care) and appoint contractors or trained staff to for regular maintenance of proper functioning of the devices and properly dispose of the residual according to requirement listed out on the product label;

- Continuously review strategic locations required for installation of MTDs and timely adjust the programme to cope with the needs;

- Carry out fogging in the scrubby areas 100 metres radius around residences with activities of adult mosquitoes to kill the adult mosquitoes once weekly;

- When carrying out fogging, the following points should be noted :

- Fogging operations should only be conducted by using ultra low volume (ULV) fogging machines.

- Concentration used for ULV application is different from other application methods.  Operators should closely follow instruction on insecticide labels for correct dosage and should only use insecticide products designated for ULV application.

- The preferable time for conducting fogging operations is early morning or late afternoon.

- Fogging operation should not be conducted under heavy rain or windy condition.

- Only water should be used as solvent for dilution of insecticides.

General points to note for departments

Member departments/Bureau of the Interdepartmental Working Group on Pest Prevention and Control and HA will be discussed through interdepartmental meetings to improve communication. All departments will be requested to strengthen the mosquito prevention and control work in venues under their purview.

Our recommendations to be given to them would be same as those as for the Operations as listed below:-

For venues with appointed pest control contractors

- Carry out regular inspections to MTDs installed to ensure proper functioning of the devices.

- Carry out weekly fogging at areas with dense vegetation and with high human concentration, e.g. sitting-out areas, parks, slopes near PHEs.

- Carry out weekly inspection to remove water accumulation and larviciding (if necessary) at potential mosquito breeding places with particular attention to surface channels, sand traps and drainage.

- Cleanse drains and gully/sand traps regularly to prevent choking.

- Dispose of articles that are able to contain water such as empty lunch boxes, cans and disused tyres.

- Cover all containers that hold water to prevent mosquito from accessing the water.

- Remove fallen leaves/disused articles in flowerbeds and slopes at close proximity to human activities regularly.

For venues without pest control contractors :

- Assign a staff for the control and prevention of mosquito in the venue.

- Cover or fill all keyholes of manholes covers.

- Promptly clearance of refuse within the properties

- Remove fallen leaves at hillslopes.

- Remove all unnecessary water collection and eliminate the source(s).

- Inspect water trays regularly and drain away water properly.

If government departments encountered problems on mosquito control, FEHD will provide technical advice on remedial action(s) to the stakeholders.

Common irregularities noted at departmental venues

Parks, sitting-out areas, playgrounds etc.

- Ponds of stagnant water

- Stagnant water trapped in tree name plastic cover / bags

- Stagnant water trapped in bamboos for holding small trees

- Stagnant water trapped in leaf axils of plants (e.g. traveller’s palm)

- Improper storage of gears / canvas cover or shade forming water receptacles and breeding ground of mosquito

- Stagnant water trapped in channels not properly drained

- Slabs for placing air conditioning unit trapped with stagnant water

- Improper storage of waste / trimmed vegetation that trap water

Public housing estates

- Unattended structure /roof top trapped with stagnant water

- Gutter of covered walkway choked with fallen leaves/debris and trap water

- Slabs for placing air conditioning unit trapped with stagnant water

- Stagnant water trapped in channels not properly drained

- Refuse hanging on the trees that traps water

- Gully traps not properly cleansed forming a mosquito breeding ground

- Piling up of junk and other articles at the refuse collection points forming water receptacles and breeding ground of mosquito

- Water logging at surface channel

Vacant government sites

- Long grass was not properly trimmed and forming habitat of adult mosquito

- Refuse trapped with water

- Stagnant water trapped in uneven ground surface

- Stagnant water trapped in low level site without drainage

- Stagnant water trapped in channels not properly drained

- Accumulation of disused articles and holding of stagnant water

Constructions sites

- Stagnant water inside water barriers forming a mosquito breeding place

- Canvas covering construction material trapped with water forming serious breeding ground.

- Long grass was not properly trimmed and forming habitat of adult mosquito

- Water accumulated in vents on surface of service reservoir

Hospitals and clinics

- Bottom of flower pots trapped with water

- Stagnant water trapped in channels not properly drained

- Fallen leaves trapped the stagnant water at surface channel

- Improper storage of gears / refuse forming water receptacles and breeding ground of mosquito.

- Water containers not properly covered and forming a mosquito breeding place

Schools and universities

- Bottom of flower pots trapped with water

- Water containers, particularly in nursery, not properly covered and forming a mosquito breeding place

- Stagnant water inside water barriers forming a mosquito breeding place

- Refuse trapped with water

For Private Premises (such as residential buildings, village houses and old tenement buildings)

For private premises and building without management office, HAD will organize and mobilize the private sector to contribute on mosquito control through different channel such as FEHPWC and DC meetings. In this aspect, we suggest HAD and concerned private parties could follow the following guidelines to control mosquito infestation.

For general public (household) :

- Change water and thoroughly cleanse the flower vases weekly.

- Avoid using base plates under potted plants.

- Take personal protection measures (wear light colour long-sleeves clothing) and apply insect repellent when carrying out outdoor activities.

- To tidy up their premises and check for any accumulation of water inside their premises.

- Maintain drainage pipes of canopy and roof regularly to prevent choking.

- Inspect canvas/plastic sheets regularly to avoid water accumulation.

- Seal the end of hollow metal bollards or rods.

- Cover metal rod holders with lids or plug to prevent accumulation of water.

- Avoid accumulation of articles on roof top